Design Project Report 2017

ELEC 2320

Alexandra Rendina C3256969

Taylor Young C3206230

26/10/2017

Contents

1.0 Main Circuit	3

1.1 Summing Amplifier	3

1.2 Voltage Controlled Current Source (VCCS)	4

1.3 Inverting Amplifiers	4

2.0 Input Detection	6

3.0 Frequency Detection	6

3.1 Multiple Feedback Circuit	6

3.1.1 Passive half wave rectifier	8

3.1.2 Comparator	8

3.2 Butterworth Circuit	9

4.0 Voltage Detection	11

4.1 1 to 4V Input Voltage detection	11

4.1.1 Window detecting circuit	11

4.2 2V Input Voltage Detection	12

5.0 Practical Considerations	13

5.1 Resistors	13

5.2 Capacitors	13

5.3 Operational Amplifiers	14

5.4 Power Supply Noise	14

6.0 Design Implementations	16

6.1 Main Circuit	16

6.2 Input Detection	16

6.3 Narrow Frequency Detection Circuit	16

7.0 Practical Results	17

7.1 Main Circuit	17

7.2 Input Detection	18

7.2.1 Frequency Detection	18

7.2.2 Amplitude Detection	18

7.2.3 DC offset	18

7.3 Narrow Frequency Detection	19

8.0 References	20

Appendix A	21

# 1.0 Main Circuit

The main circuit, Figure1, consists of a buffer, inverting summing amplifier, voltage controlled current source, an opto-coupler, a series capacitor and two inverting amplifiers. The function of the main circuit is to amplify the given input signal of

2+A sin2πft Volts

and remove the DC offset such that the resulting output is

2A sin2πft Volts

for Amplitude, A, and frequency, f. (See Appendix A figure 21 and 22 for response)

Figure 1: Main Circuit

The signal is first summed by the amplifier, this results in the input voltage we require. Opto-Couplers are current controlled devices. For the LED to produce the infra-red light to be received by the photo diode, a minimum current of 10mA is required. The LED is controlled with changing current, so a Voltage controlled current source is used in the circuit to provide a supply current to the opto-coupler. The photosensor detects this infra-red light and generates a voltage. A series capacitor is then implemented to remove the DC offset. The two amplifier circuits are then used in combination to achieve a signal of amplitude 2A and inverting the signal such that the orientation matches that of the input signal. (see Appendix A Figure 2)

## 1.1 Summing Amplifier

Figure 2: Summing Amplifier

A minimum of a 2V DC offset is required as the input for the Amplifier to ensure that the sine wave output is always positive, thus a value larger is used to avoid cropping signal.   To calculate Vout, the circuit was analysed to achieve:

Vout=(1+R1R2)Vin 	.

Values for resisters R1 and R2 were decided to achieve a voltage output which sums the two inputs with an amplification of 1.

## 1.2 Voltage Controlled Current Source (VCCS)

For the Opto-coupler we require a changing intensity of the LED light. To achieve this an alternating current source is required, since the intensity of an LED is proportional to the current through it. By implementing a voltage controlled current source, we can supply a sinusoidal current wave from a sinusoidal voltage.

As the Opto-couple has a non-linear output to the current through it, it is beneficial to work in a small, mostly linear, section of the curve. It has been determined that 50-90mA is a suitable range to work in for the QED123. Analysing the circuit seen in figure 3, assuming ideal op-amp characteristics i.e. AOL= ∞, I+=I-=0

Iin= Il

Il= VinR16

Figure 3: Voltage Controlled Current Source

## 1.3 Inverting Amplifiers

The DC bias can now be removed with a series capacitor (decoupling), this is essentially a high pass filter. The two inverting amplifiers are used to increase the signal to double the amplitude of

2A sin2πft Volts

Amplifier Gain to calculate resistor ratios

Figure 4: Inverting Amplifiers

# 2.0 Input Detection

As part of the design brief the circuit requires that when certain input conditions are met, an LED is activated. For this feature the following three conditions must be met;

- Frequency between 20Hz – 20kHz

- AC amplitude 1 – 4 volts

- DC bias of 2 volts.

To satisfy all three conditions at once some hardware implemented logic was needed. Two options were explored, a set of Zener diodes on each conditional output, or an op amp comparator with a set reference.

The latter was selected as the better option as access to components guided the decision. This circuit works by comparing the potential of the three conditional outputs to a reference voltage, as shown in figure 5, less than the combined outputs by greater than only two.

Figure 5: AND Logic

The following sections address in detail the frequency and voltage detection circuits.

# 3.0 Frequency Detection

The specifications require two separate frequency detections a wide band pass of 20Hz to 20Khz and a small band pass 970Hz to 1030Hz. From the characteristics required of each of the band passes, we chose to use a Sallen-Key topology, more specifically a Butterworth filter, for the wide and a Multiple Feedback for the narrow band pass circuit

## 3.1 Multiple Feedback Circuit

The layout of the filtering is pictured in figure 6. The circuit consists of a voltage follower, the multiple feedback bandpass, rectifiers and comparators. The voltage follower is a buffer used to isolate the circuit to prevent interference with other filters that could potentially result in a ghosting effect. The signal is then passed through the Multiple feedback filter, where frequencies outside of the band are attenuated. The two rectifiers are used to covert the AC signal to DC at the node after the buffer and at the node after the multiple feedback filter. The comparator is then employed to compare the two voltages. The V- acts as the reference voltage and if V+, the output of the filter is +Vcc. If the reference is greater than the input however, the output is negative Vcc and thus the red LED will not turn on.

Figure 6: Multiple Feedback Filter

The Multiple Feedback filter was implemented for the narrow band pass. This topology was chosen because of the opportunity to control the centre frequency, the gain and the quality factor. In comparison to the Sallen-Key circuit, the maximum to minimum component values is much higher and in addition, the multiple feedback filter inverts the phase. This effectively creates a phase shift of 180 degrees however, the response very effective and the phase change does not matter for our filtering purposes. The components for the multiple feedback bandpass were calculated using the equations in figure 7 corresponding to the circuit layout in figure 8, in an excel spreadsheet to optimise values (see appendix A). (See Appendix A figure 24 for response)

Figure 7: Multiple Feedback component equations

Figure 8: General form of Multiple Feedback Circuit

### 3.1.1 Passive half wave rectifier

After the signal is filtered it is then rectified such that only the positive half of the signal is allowed through to achieve a DC current as opposed to the AC output of the bandpass. A half wave rectifier, see figure 9, is used for this purpose. The rectifier consists of a diode, a capacitor and a resistor. When low Ac voltages are applied, the diode becomes forward biased however during the negative region of the AC signal it acts in reverse bias preventing the majority of signal from passing. The purpose of the resistor is to consume the small current generated by the diode. The capacitor is then used to convert the pulses into a smooth DC current. The advantages of the half wave rectifier are the simplicity of its design and the isolation it provides from input.

Figure 9:Halfwave Rectifier

3.1.2 Comparator

Two separate rectified DC signals are then passed through a non-inverting comparator. One of these signals is the filtered input, the other is a reference and has a voltage value such that the voltage corresponding to the boundaries of the band pass filter is slightly greater.  From inspection it can be gathered that Vd= (V+) - (V-).  When the reference voltage connected to the negative input of the op amp is greater than the DC signal at the positive terminal, Vd is negative and thus Vout will saturate at –Vsat. When the filtered signal, the positive input, is greater than the reference, the op amp will saturate the signal at positive Vsat. If the signal coming out of the comparator is not positive, the LED will not work because it requires a positive voltage.

## 3.2 Butterworth Circuit

A Butterworth filter of Sallen-Key topology was used for the wide bandpass of 20Hz to 20kHz because of its overall reliable performance. The Sallen-Key topology is a reliable configuration as it depends less on the performance of the op-amp because it is connected as an amplifier rather than an integrator minimising bandwidth requirement thus we could design a filter for higher frequencies. An added advantage is the small ratios between components making resistor and capacitor values for the practical construction easier to achieve.  The Butterworth was chosen over other Sallen-Key filters Bessel and Chebyshev because of the ripple free magnitude response in the pass band as well as a better pulse response than the Chebyshev and a better attenuation rate than the Bessel.  A third order low pass Butterworth and a third order high pass Butterworth were cascaded to achieve the desired result. Each third order is compiled of an active second order and a passive first order.

Figure 10: Butterworth Filters

In figure 10, the circuit contains high pass and low pass components of the filter preceded by a Voltage follower to separate the band pass from the other circuit components. To make the calculations for the component values of the Butterworth filter simple, an equal component value topology was used. Equations and values were put in an Excel spreadsheet to allow for variation depending on available components. (See Appendix A figure 23 for response)

Coefficients a1 and b1 were obtained from the Butterworth table (see appendix A, figure 19 )
for an n value of order three.

# 4.0 Voltage Detection

The brief specifies two separate instances where voltage detection must be used. The input must be checked to determine whether it is the specified 2V and the value of the amplitude must be measured to be between 1 and 4 volts. For both cases, the layout of the circuit is similar in that the input is put through a buffer, the signal is rectified and then a comparator is used to compare the voltage to a reference to determine whether it is valid.

## 4.1 1 to 4V Input Voltage detection

The specified voltage input is between 1 and 4 volts. To detect this, we are using the circuit in figure 11 that consists of a rectifier and a window comparator.  The rectifier is a simple passive half wave rectifier as used for the frequency detection, (see section 3.1.1).

### 4.1.1 Window detecting circuit

A window detecting circuit consists of two op amps acting as an inverting and a non-inverting comparator. This design is efficient as it decreases the number of op amps required to detect the voltage by checking whether the input is within the threshold rather than checking each limit separately. As in figure 11, the input (vin+) is put into the V+ of one op amp and the V- of the other (vin-). The unused V+ terminal is then the upper voltage limit and the V- the lower limit, in our case 4 and 1 volts respectively. The window comparator works such that whenever the signal is either higher or lower than the bounds of the reference, the op amps will output a signal at –Vsat.

Figure 11: 1 – 4 Volt Window Comparator

## 4.2 2V Input Voltage Detection

Figure 12:2 Volt Detection

The specified DC offset of 2V must be detected to confirm the input signal is valid. Like the 1 to 4 Voltage input detection, a window comparator, pictured in figure 12, is used. However, the acceptable range for voltage is much smaller.

# 5.0 Practical Considerations

- Resistors are not exact and are only made to a specified tolerance for a value, although this is usually a very small tolerance.

- Capacitors are not exact and are made to a specified tolerance for a value, although this tolerance is usually very small

- Operational Amplifiers have a few non-ideal characteristics including: Slew Rate, Output Current, Output Voltage Swing, Input Bias Current, Input Offset Current, Input Offset Voltage, and CMMR just to name a few.

## 5.1 Resistors

The resistors in use are made to a very small tolerance, for most applications with values below 100k this tolerance is not significant, however in some positions this tolerance is too much to accurately create circuits.

Resistors are only made in specific values and when the exact resistance is unavailable, a close value can be used. As above, for most applications this does not affect the performance of the circuit.

To overcome both these issues, resistors can be replaced by potentiometers, allowing fine tuning of the resistance value.

In figure 4 resistor (R19) was replaced by a 1kohm resistor in series with a 100kohm potentiometer. The 1kohm resistor limits the output voltage equal to the input voltage and the potentiometer allows fine adjustment of the total amplification of the signal, to reach the required 2A.

## 5.2 Capacitors

As for resistors, capacitors are also made to a 1% tolerance, and for most applications this does not affect performance. Capacitors also come in specific values, of which needs to be considered in circuit design and accounted for. However, when building frequency filters, the value of capacitors tunes the cut-off frequency of the filter and therefore an incorrect capacitor value can cause bandwidth issues.

To overcome this problem in the filter design, the available capacitor values can be used to calculate the resistors, instead of calculating the capacitor values based of predetermined resistors, as resistors come in a wider range of values.

The original models for both filters contained capacitor values calculated off known resistor values, however after examining the equipment available the circuit was recalculated for available values of capacitors, whilst keeping the overall design of the circuits.

## 5.3 Operational Amplifiers

It was determined that most of the practical considerations listed were not a problem in the applications which the op amps are being used, however slew rate and output current are both of importance.

The slew rate of an IC describes the maximum rate of change of the output voltage with reference to microseconds, essentially due to factors inherent in the operational amplifier internal circuitry. If the slew rate is exceeded the output voltage can be distorted.

The specifications of the LM741 and LM747 op amps have a slew rate of 0.5V/us. For most applications in this circuit the slew rate is not exceeded, however in the final stages of the signal processing the op amp is required to output 16Vpk-pk up to 20kHz. At this frequency and amplitude, the slew rate would be defined as;

Figure 13:Maximum Slew Rate

Gradient m=1.0433V/us

To overcome this limitation a better suited op amp was used, the OPA551. This allows 15V/us which is more than enough for this application.

In addition to the slew rate increase, the OPA551 has a higher specification of output current, at 200mA as opposed to the 25mA of the LM741. This output current is the maximum current the op amp can output continuously. This becomes important when paired with the QED123 photodiode, as the current range of the diode is 10mA to 100mA. If restricted to the smaller output of the LM741, the nonlinear properties of the diode would create issues with distorted signals. However, with the OPA551, the range can be more carefully selected to improve the output response.

## 5.4 Power Supply Noise

An inherent aspect of working with any power supply is the noise that is transmitted through the power rails, see figure 14. This noise can come in two main forms, high and low frequency. Induced AC noise at 50Hz is combated by adding 10uF capacitors across the two power rails, and the higher frequency noise is counteracted by also placing 100pF capacitors across the power rails.

Figure 14: Power Rails with capacitors

In addition, some op amps were fitted with a decoupling capacitor of 100nF between the supply voltages and their respective op amp terminal as shown in figure 15. This is used to reduce noise of the incoming power.

Figure 15: Decoupling Capacitors on Op-Amps

# 6.0 Design Implementations

Whilst implementing the simulated design of the circuit, several issues arose that were inconsistent with the results in the simulation. These were due to some inaccuracies due to the idealised nature of the simulation as well as imperfection in component values and behaviours.

## 6.1 Main Circuit

When measuring the output response of the opto-coupler, the crests of the sine wave were cropped. After investigation the issue, the current into the opto-coupler was noted to be close to the 100mA limit of the device. The added DC offset was reduced resolve the issue by lowering the current offset of the VCCS and thus total current to around 85mA. This resulted in a better output response from the circuit.

In simulation, this circuit benefited from a 100uF series capacitor to remove the DC bias. The size of capacitor controlled the frequency response at low frequencies and for values around 10uF the response was attenuated at <20Hz. As the value was increased, it was observed that the attenuation reduces with diminishing returns. For an ideal response, a 100uF capacitor was required, however due the physical size and need of a 100uF bipolar electrolytic capacitor, it was determined to be inconvenient and unsuitable. A compromise was made between diminishing returns and a perfect response resulting in a 20uF capacitor being used.

The circuit also required the removal of the 2 volts DC bias, however, the output of an op amp is not perfectly tuned to 0 volts DC. This caused issues with the method being used for bias removal, as this process was done before the last op amp, leaving a bias of around 500mV. To improve this a passive low pass filter can be added to the final output without effecting the gain of the remaining system.

## 6.2 Input Detection

The input detection circuit had many design changes when implementing the circuit. Interference from the three parallel detection circuits caused issues with the input voltage widows comparators. This resulted in incorrect reference values of these comparators, preventing the output from going high. To resolve this issue potentiometers were used to fine tune the reference values.

## 6.3 Narrow Frequency Detection Circuit

To make a narrow frequency filter work precisely, specific valued components are required, however this is impractical because resistors and capacitors are not available for all values. To overcome this issue, the filter was chosen to have lightly coupled parameters between components. The resistor (Figure6 R16) was replaced with a series 100ohm resistor with a 100ohm trim pot. This allows precise centre frequency adjustment, with minimal effect on the Q and amplitude of the output.

# 7.0 Practical Results

After building and refining the designed circuit, the following section shows the practical results achieved, (see Appendix A figure 20 for data).

## 7.1 Main Circuit

The main circuit performs as per the design specifications, the following figure 16 shows the circuits amplification at 4 volts input signal and specified 2-volt DC offset.

Figure 16:Gain vs Frequency of Main Circuit

## 7.2 Input Detection

As required the circuit can detect correct range of amplitude of the AC signal, DC offset and frequency of input into the system.

### 7.2.1 Frequency Detection

The 20 – 20kHz filter circuit has a maximum of 3 gain in the middle of the frequency range, attenuating to -3dB at 12.34Hz and 20kHz as evidenced by the plot figure 17.

Figure 17:Gain vs Frequency of 20-20kHz Filter

### 7.2.2 Amplitude Detection

The cut-off voltage for the 1 to 4-volt window comparator sits at 1.8V and 2.2V, this is within practical expectation of the given design

### 7.2.3 DC offset

Similarly, to the 1 to 4-volt window, the 2-volt detector can read between 0.8V and 4.2V, this is close to the expected response of the system.

## 7.3 Narrow Frequency Detection

The 970 – 1030Hz filter circuit has a maximum of 3.6 gain in the middle of the frequency range, attenuating to -3dB at 980Hz and 1060Hz as seen in figure 18 below.

Figure 18:Gain vs Frequency of 970-1030Hz

# 8.0 References

Linear Circuit Design Handbook Chapter 8

by Analog Devices Inc., Engineeri, Hank Zumbahlen, Analog Devices Inc.,

Electronics, F. (n.d.). What is an Optocoupler? Retrieved September 26, 2017, from http://www.futureelectronics.com/en/optoelectronics/optocouplers.aspx

[electronicsNmore]. (2011, November 12). Understanding Optocouplers [Video File]. Retrieved from https://www.youtube.com/watch?v=-gkrzPGqjOg

H. Z. (2008). Chapter 8 Analog Filters. In Linear Circuit Design Handbook . Newnes/Elsevier.

Op Amp Comparator. (n.d.). Retrieved October 6, 2017, from http://www.electronics-tutorials.ws/opamp/op-amp-comparator.html

Optocoupler Tutorial (n.d.). Retrieved September 26, 2017, from http://www.electronics-tutorials.ws/blog/optocoupler.html

# Appendix A

Butterworth Filter Coefficients

Figure 19:Butterworth Filter Coefficients

MBF excel sheet:

| Multiple Feedback |  |
| --- | --- |
| R1 | 80381.28439 |
| R2 | 217.6170338 |
| R5 | 241143.8532 |
| K | 0.00013823 |
| H | 0.09 |
| Q | 16.66666667 |
| FM | 1000 |
| BW | 60 |
| C3 | 0.000000022 |
| C4 | 0.000000022 |
| Fo | 1000 |
| A0 | 1.5 |

Figure 20:Date for Practical Results

Practical Results of circuit

Figure 21:Main Circuit Output Bode Plot

Figure 22:Vin vs Vout at 1kHz

Figure 233: Butterworth Filter Output Bode Plot

Figure24: Multiple Feedback Filter